Weaving and Knitting Operations Level- II

Based on March, 2022, Curriculum Version I,

Module Title: Performing Handloom operation

Module Code: IND WKO2 M 07 0322

Nominal duration: 107Hours

Prepared By: Ministry of Lobar and Skill

Month, Year

Addis Ababa, Ethiopia

# Acknowledgement

Ministry of Labor and Skills wish to extend thanks and appreciation to the many representatives of TVET instructors and respective industry experts who donated their time and expertise to the development of this Teaching, Training and Learning Materials (TTLM).

# Table of Contents

# Acronyms

# Introduction to Performing Handloom operation

A handloom is a type of loom used to weave cloth or patterns on cloth and is operated manually without the aid of electric power. What started as a mechanism to furnish necessities has become a specialized tool for the handcrafts man specializes in furnishing art and luxury fabrics. The growing textile industry has created a vast demand for the handloom weavers which shaped the much of the 19th century cotton industry. No one exactly knows how or when weaving began, but the idea of weaving clearly predates the loom by at least thousands of years. However, the once commodity-generating sector is dying owing to the rise of more efficient modern looms such as power looms

Weaving is a process of fabric production in which two distinct sets of yarns are interlaced at right angles to each other to form a fabric or cloth.  The lengthwise yarns are called the warp yarn and the widthwise yarns are called the weft yarn

# Module units

- Yarn Preparation

- Pre-looms activities

- Weaving design

- Weaving process

- Product quality

- Handloom weaving related operations

- Handloom materials, tools and machines

# Learning objectives of the Module

At the end of this session, the students will able to:

- Prepare Yarn for weaving process

- Check and ensure the pre-looms activities

- Analyze weave design

- Carry out the weaving process

- Confirm the specified quality

- Identify handloom weaving related operations

- Maintain and handle materials, tools and machines in the work area

# Module Learning Instructions:

- Read the specific objectives of this Learning Guide.

- Follow the instructions described below.

- Read the information written in the information Sheets

- Accomplish the Self-checks

- Perform Operation Sheets

- Do the “LAP test”

# Unit one: Preparation of Yarn

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Preparation of work area for warp and weft yarns - Preparation of Pirn - Preparation of warp yarn This unit will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Prepare work area - Prepare Pirn (weft yarn) for weaving process. - Prepared Warp yarn.

## Preparation of work area for warp and weft yarns

### Manual hand loom

Weaving is the most basic process in which two different sets of the yarns and threads are interlaced with each other to form a fabric or cloth. One of these sets is called warp which is the length wise yarn running from the back to the front of the loom. The other set of crosswise yarns are the filling which is called the weft or the woof.

A hand loom is a simple machine used for weaving. In a wooden vertical-shaft looms, the heddles are fixed in place in the shaft. This loom is powered by hand. The warp threads pass alternately through a heddle, and through a space between the heddles (the shed), so that raising the shaft raises half the threads (those passing through the heddles), and lowering the shaft lowers the same threads-the threads passing through the spaces between the heddles remain in place. It is a manual operating system. Shedding is done by pressing the treadles pedal and picking and beating is done   manually. Hand looms yield less production compared to power loom. It can be operated at slow speed only compared to high speed of power looms. Initial investment is very low compared to power loom.

Figure 1.1: Principles of weaving

### preparation of work area for warp and weft yarn

Almost all fabrics, whether woven or knitted, are produced from spun thread. However, this thread or yarn, produced by the spinning operation, either by a hand spinner using the simplest drop spindle or spinning wheel, or spun on the latest automated spinning frame, is rarely in a form suitable to be used immediately for producing a fabric. A broad range of tasks must be undertaken before the actual fabric production is reached.

"Yarn Preparation" is concerned with the preparation of yarns which have been spun from staple fibers, and describes the post-spinning processes prior to fabric manufacture, such as yarn doubling, winding and spinning. Covers the operations to consider when producing a fabric by even the most basic of techniques. The information on basic processes, machinery and equipment should assist those people involved in the production of fabrics by showing ways to improve the quality to the end product. Includes a list of suppliers, sources of further information and a reading list. This is a valuable book which fills the need for a practical manual specifically about yarn preparation. Much has been published about the main textile areas of yarn manufacture and fabric production (both weaving and knitting), because yarn preparation tends to be a series of link operations, there have been very few books specifically directed to this area of processing. The "Small-Scale Textiles" series aims to present basic information about all aspects of small-scale textile manufacture from raw materials to finished products, and will be of use to fieldworkers, development agencies, and those starting small-scale manufacture or attempting to improve or extend manufacture.

So, before starting the loom operation the weaver should prepare some activities regarding to work area are:

- Yarn is the basic building block in weaving. However, in practice, the condition of yarn produced on the spinning machine is not always good enough to be used directly for fabric formation.

- Both filling yarn and warp yarn to be further processed for efficient fabric formation. These preparatory processes are called weaving preparation.

- Warp and filling yarns are subjected to different conditions and requirements during weaving. Therefore, preparation of warp and filling yarns is different.

- Warp yarn is subjected to higher stresses which require extra preparation.

- The filling yarns are not subjected to the same type of stresses as the warp yarns and thus are easily prepared for the weaving process.

- Winding is the major process and common for both filling and warp preparation; warp winding process, warping and weft yarn winding process, pirn winding or quill preparation.

Before starting the loom, the weaver should ensure optimum tension in the warp sheet so as to avoid wrong beat up and to avoid the yarn breakages.

Correct the fabric defects like wrong drawing, wrong denting, end out, double end etc., immediately and also ensure that the other fabric defects too are corrected at the earliest, before continuing further production.

Clean the machines & work area, so as to ensure good working atmosphere, while cleaning ensure not to damage the fabrics in the looms as well as in the adjacent & opposite looms

Operate the loom without " starting mark or crack"

Ensure that the loose threads are hanged at higher length (not more than 4 mm) and trimmed, after attending to the warp breaks.

Always ensure that the correct weft yarn is used

See that the weft yarn is completely used, without giving room for additional wastage of raw materials.

Ensure correct quality of thrums are there & see that the same are properly tied

Check the knotted loom for knotting quality etc.

Remove double ends if any.

Ensure that no raw material/ cloth or any other material is thrown under loom.

Check the reasons for the frequent warp/ weft breaks and correct in the loom

## Pirn or weft yarn Preparation

### Filling or weft yarn

Weft yarn is a set of yarn which lays across the width of the fabric and called filling yarn or quill. There are different shape and sizes of weft yarn prepared or wound on its suitable package according type weaving loom. For manual handloom weaving, pirn or weft yarn wounded package; and for shuttles weaving, cone package is prepared.

Weft winding is the process of transferring weft yarn on the small bobbin called Pirn, able to use in the shuttle of a loom. A tapered bobbin having weft yarn on it is known as Pirn. It is used on end feed shuttles.

Because the weft does not have to be stretched on a loom the way the warp is, it can generally be less strong. It is usually made of spun fiber, originally wool, flax and cotton, today often of synthetic fiber such as nylon or rayon.

The weft is threaded through the warp using a "shuttle", air jets or "rapier grippers". Handlooms were the original weaver's tool, with the shuttle being threaded through alternately raised warps by hand.

For horizontal or weft yarn preparation, traditionally, charka is used. By the fingertips, correct tension is given to the yarn. A hank of yarn is wound onto a small bobbin called “pirn”. The weft yarn wound on pirn is then inserted into a shuttle (a device used in weaving to carry the weft thread back and forth between the warp threads.)

The pirn differs from bobbins in that the pirns are tapered. There two types of pirns:

- Wooden pirn

- Plastic or nylon pirn

Figure 1.2a: Wooden pirn                                                 Figure 1.2b: Plastic or nylon pirn

Types of pirn winding machines:

- Ordinary or non power/ manual Pirn winding machine

- Automatic Pirn winding machine

- Cop winding machine

Figure1.3: Ordinary Pirn winding machine

### Pirn preparation

Weft is the yarn that goes across width of a woven fabric.

- Weft yarn undergoes pirn winding or quill.

- Quill is bobbin on which filling yarn is wound.

- The bobbin is placed inside the shuttle.

- The machine that are used to wind the quills are called as quillers /pirn winding machine.

- This process of quill winding is gradually disappearing with the advent of shuttle less loom

### Yarn passage in pirn winding machine:

The yarn package (cone or cheese) is mounted on the cone holder. The yarn coming from the package passes through a thread guide. Now, this yarn passes through a yarn tensioner. This tensioner imparts required tension to the yarn and helps to regulate the yarn winding density on Pirn. The yarn again passes through another thread guide now. The yarn now passes through a yarn traverse guide. This traverse guide makes to and from movement during the winding process. Yarn finally gets would upon the pirn.

Figure 1.3: Schematic diagram yarn passage

## Preparation of Warp Yarns for Weaving

### Warp yarn

It is lengthwise or longitudinal yarns of the fabric which held stationary in tension on a frame or loom while the transverse weft (sometimes woof) is drawn through and inserted over and under the warp. A single thread of the weft crossing the warp is called a pick. Terms vary (for instance, in North America, the weft is sometimes referred to as the fill or the filling yarn). Each individual warp thread in a fabric is called a warp end or end.

The warp is the set of yarns or other things stretched in place on a loom before the weft is introduced during the weaving process. It is regarded as the longitudinal set in a finished fabric with two or more sets of elements.

The term is also used for a set of yarns established before the interworking of weft yarns by some other method, such as finger manipulation, yielding wrapped or twined structures. Very simple looms use a spiral warp, in which the warp is made up of a single, very long yarn wound in a spiral pattern around a pair of sticks or beams.

The warp must be strong to be held under high tension during the weaving process, unlike the weft which carries almost no tension. This requires the yarn used for warp ends, or individual warp threads, to be made of spun and plied fiber. Traditionally wool, linen, alpaca, and silk were used. However, improvements in spinning technology during the Industrial Revolution created cotton yarn of sufficient strength to be used in mechanized weaving. Later, artificial or man-made fibers such as nylon or rayon were employed.

Figure 1.4: warp yarn and weft yarn

### Warp yarns preparation for handloom weaving

- Preparation of warp is important because warp has to undergo various stresses like extension /contraction, rotation while twisting/untwisting and abrasion with yarn as well as metal.

- Also, faster weaving speed and use of insertion device are apart from shuttle have increased the necessity of warp preparation before weaving.

- The preparation of warp is more demanding and complicated than that of filling yarn.

- Each spot in a warp yarn must have uniform properties with sufficient strength to withstand stress and friction during weaving.

- The no. of knots should be kept to a minimum.

- Yarn on the warp sheet must be parallel to each other with equal tension.

- Warp preparation involves winding, warping, slashing and drawing in or tying in

- Good warp preparation is an essential requirement to achieve optimum efficiency and fabric quality

There are very important process in preparation of warp yarns:

- Bobbin winding and Warping

With the help of reeled yarns, the dyed or undyed yarn hank gets converted into a linear thread form and wound on the bobbin. This process enables laying out of yarn lengths for weaving.

Warping is done, which is the parallel arrangement and winding of warp yarn from bobbin to the warp beam. Traditionally, the weavers use a big rotating drum as warp beams and decide the width and length of the final fabric. These drums help them in counting the number and colour wise grouping of yarns. Also, the dimensions of the warp are decided by the weaver at this stage.

- Sizing of warp yarns

Post warping, the warp yarns are stretched out for size application. Sizing material or starch is applied to add strength and lubricate the yarn. This crucial activity is called "sizing". Natural sizing material like rice, maize, wheat flour, or potato starch is used depending on the region. After the application of the sizing paste onto the stretched yarn, special brushes are used to spread and dry the starch on the yarn. This starch is removed only after two to three washes of the finished product.

- Dressing and Winding the warp yarns

Before the size applied warp is loaded onto the loom, the warp yarns are aligned and separated to facilitate smooth weaving. The aligned and starched yarns are carefully wound around a wooden beam and carried to the loom.

- Attaching Warp Yarns on Loom

Each warp yarn is drawn through heddles and reed and finally tied on both front beam and back beam. According to a pre-determined weave plan, yarns are passed through heddles which separate the warp yarns into two sections between which the weft yarn (horizontal/width-wise yarn) passes

Fig manual Warping machine

# Unit two: Pre-looms activities

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Setting up of weaving loom - Pre-loom activities - Product requirements This unit will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Identify Weaving loom parameters - Perform Pre-loom activities - Check loom setting against product requirements

## Setting up of Weaving loom

### Weaving Loom

A loom can be defined as any device used to weave thread or yarn into cloth, a rug or a tapestry. At its core, a loom holds one group of parallel threads under tension (the warp) while enabling the weaving of another set of parallel threads (the weft) perpendicular to the threads under tension. The pattern of the weave is typically determined by the manipulation or movement (up or down) of the warp threads.

As they weave, the weaver lifts or lowers some of the warp threads to form an opening between them called a shed. The weaver then pushes the weft through that opening using a tool called a shuttle. Except for the most basic of looms, such as frame looms, all looms have some method for creating sheds. For example, on multi-shaft looms, warp threads are lifted or lowered because they are threaded through heddles that hang on frames called harnesses. When the weaver uses treadles or levers to lift or lower the harnesses, the warp threads threaded through heddles on those harnesses go up or down and create a shed. On simpler looms (inkle looms, backstrap looms, and rigid-heddle looms), the heddles are moved up or down manually to create the shed.

A weft-carrying shuttle can be as simple as a stick wrapped with thread or can be a fairly technical flying shuttle that zooms across the warp with the quick flick of a cord. In either case, as a shuttle moves through the shed across the warp, it leaves a trail of weft. Each pass of a shuttle through the shed is called a pick. After each pick, the weaver changes the shed by changing which warp threads are lifted or lowered and places the pick using a part of the loom called a beater that holds what is called a reed that resembles a very large comb in a frame. Placing a pick is called beating, except for the case of a heavy rug where placing is a better description.

A hand loom is a simple machine used for weaving. In a wooden vertical-shaft looms, the heddles are fixed in place in the shaft. This loom is powered by hand. The warp threads pass alternately through a heddle, and through a space between the heddles (the shed), so that raising the shaft raises half the threads (those passing through the heddles), and lowering the shaft lowers the same threads-the threads passing through the spaces between the heddles remain in place. It is a manual operating system. Shedding is done by pressing the treadles pedal and picking and beating is done   manually. Hand looms yield less production compared to power loom. It can be operated at slow speed only compared to high speed of power looms. Initial investment is very low compared to power loom

Figure 2.1: weaving loom

### Important parts of hand loom

It is very important to know about the parts and accessories of the handloom and their function. Without knowing of their function you can't perform or work on a handloom. So it is a must to know, the major component of list is given below.

- Warp Roller: The warp roller which consists of the lengthwise yarns is located at the back of the loom & it releases the warp yarn to the weaving area of the loom as needed.

Figure 2.2: Warp roller, handle, Back beam

- Handle: The handle is provided on the warp roller to tighten or loosen the warp sheet which ultimately changes the tightness factor of the fabric.

- Back Beam: To maintain the constant tension and proper angle to the warp sheet

- Heddles: it is a frame to hold the Heald wires. Heald wire is a wire with a hole or eye in its center through which a warp yarn is threaded. In some cases Heald wires made out of cotton or   synthetic yarn are also used.

Figure 2.2: Heddles

- Beater/Reed: It is a comb type device.  The function of reed is to maintain individual ends of the warp sheet parallel and also to maintain the width of the fabric This is inevitably a combination made up of metallic wires set vertically in a frame. The spaces between the wires are known as dents. There is a beater which has a Reed frame mounted on it. The weaver holds the beater and gives beater a to and fro motion for beating the last pick to the fell of the cloth.

Figure 2.3: Beater

- Cloth roller: It is located at the front of the loom. After completion of weaving the woven fabric is wound on it.

Figure 2.4: Cloth roller

- Treadles: These are located at the bottom of the loom and are designed to control warp shed formation by controlling the up and down movement of the heddles. The weaver presses the treadles by their feet for shed formation. The shed on a handloom is controlled manually by giving proper movement to the treadles using foot.

Figure 2.5: Treadles

- Shuttle: It is a device that carries weft yarn to and fro across the fabric width. It contains a spool  of a  weft yarn called a bobbin. .

Figure 2.6: Shuttle

### Weaving Loom Parameters

The loom parameters like correct warp tension, proper opening of shed, reed movement etc, is to be checked to ensure whether the loom is compatible for weaving or not. If not, the weaver has to make adjustments in the above as required and should take precautions, to minimize or avoid the yarn breakages. It is the weaver’s responsibility to check the shed opening by false picking and reed movement by false beating.

Pirn Winding: A simple machine shown in the figure below is used for pirn winding in handloom. There is a wheel and a metallic shaft, which are connected with each other with the help of rope for transferring motion. The hank is mounted on the wheel and the pirn is mounted on the shaft. The yarn from the hank is transferred on to the pirn by rotating the wheel. Precautions: While winding care should be taken and ensure:

- Uniform tension is maintained till the pirn get filled.

- The groove of the pirn should be kept empty

- The yarn should be filled on the pirn to optimum level

## Pre-loom activities

Long before the handloom weaving process of fabric can actually start, the yarns for setting up the loom is prepared. This preparatory phase starts from choosing the yarn and its quantity, goes through yarn Dyeing, Washing, Winding Yarn And Warping/Drumming, and continues all the way to drafting and denting. The pre-loom phase is time-consuming as well as takes a lot of effort. It can take 1-2 weeks for the fabric yarns to be prepared in the pre-loom phase.

The process begins with calculating how much yarn is needed per colour (depending on the planned width, length and pick count).

Pre-loom activities of handloom weaving processes are:

- Sorting of Yarn

- Dyeing:

Dyeing for handloom is normally done in and around weaving villages. The required quantity of yarn is sent for dyeing in the desired colors (which have been tested and approved). The dyed yarn is given a wash to remove excess dye. The dyed yarns are then dried after which it is ready for the next step.

- Winding:

Yarn in the hank form is wound on to bobbins in this process. This is the first step in transforming the yarn from the hank form to a linear form.  Dyed hank yarn is wound on to bobbins with the help of charkhas. This process enables the laying out of yarn lengths for weaving. Bobbin winding is done by the weaver.

Then the dyed yarns are wound onto bobbins from ring bobbins, hanks, cones, etc using reel or is called winding. The main purpose of winding is to form a single yarn package suitable for the next operation. It is also used to transfer weft yarns into bobbins or pirns to be inserted into shuttle.

- Warping:

Is a process aiming at preparing weavers beam to be set up on weaving machine. From the bobbins on the creel frame, the threads are wound on to a huge drum according to the length and width of warp in a desired sequence. Then the warp threads altogether transferred onto a weaver’s beam by unwinding the drum.

Figure2.6:Weft  winding

Figure2.7: warp winding

- Drafting

Is when each individual warp yarn is pulled out through the heald eyes on the heald wire.

Drafting is known as the selection of heald frames or harness for individual warp threads according to the design.

Figure 2.8 warp denting

Denting

Is where the warp thread is pulled out through reed dent as required reed plan and this determines more accurately the width of fabric and ends per inch.

After denting process is complete, the warp ends are fastened to the cloth beam and the process of weaving can start.

Street Sizing- The warps are stretched out onto two beams and natural adhesives are applied to add strength to the yarn and lubricate to withstand the rigors of weaving. In most handloom centres, rice starch / gruel is mixed with coconut / groundnut oil and applied as "size" material.  Sizing is carried out  by  weavers  or  specialists  in  the  village.  Since  this  activity  is  done  on  the  street,  it  is  called  "street sizing"

Tie up:

Once the weaver finishes the drawing in and denting, the next process he/she has to do is to tie the healds with the treadles.

Attaching the Warp onto the Loom- Individual warp threads  are  drawn  through  heddles  taken  through  a  set  of  reeds  and  tied  onto beams  located  on  both  ends  of  the  loom.  The heddles separate the  warp  into  two  sections  which  allows  the  weft  threads  to  pass between  them  easily .  Checks and stripes are created by segmenting the warp and weft yarn.  For  motifs,  looms  are  equipped  with "dobbies"  or  "jacquard"  cards  which help  in  lifting  segments  of  warp yarn  into  the  weft. Heddles are made out of rods or cords, each with an eye through  which the  warp thread  is  drawn.  Reed is a comb like frame that pushes the weft yarn firmly  against  the  finished cloth after each insertion.

## Product Requirements

In modern looms, achieving better efficiency with good productivity because provisions are given with Microprocessor based controls so weaving team can analyze following details in time and action prevents the losses at Grey stage.

- More Warp Breakages

- More Weft Breakages

- Stoppages due to Mechanical reasons

- Idle Stops in terms of Breakage per Hour

- Time taken by weaver to attend the loom

- Maintenance / Lubrication Schedule

- Production evaluation in terms of Loom Efficiency, Woven Meters, Picks Inserted, Total loom running hours etc.

- Control on piece length.

- Graphical analysis to control cost of production, various defect occurrence during running loom, etc.

- Weave change is possible without any mechanical changes.

- PPI can be changed from Microprocessor.

- Less wear and tear as Reed Vibrations, loose parts may be recognized from loom display before break down.

# Unit Three: Weave Design

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Weave design - Types of weave design This unit will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Select appropriate design - Analyze weave design

## Weave design

The handloom can be used for making the complicated designs with the help of dobby and jacquard. Examples: damask, weft back cloth, patent satin etc. “All products which can be produced on power looms can also be produced on handlooms. But there are many products which can be produced by handloom only.”

A fabric weave refers to the way the warp and weft yarns are interlaced in a fabric. In the most basic of fabric weaving, the threads (lengthwise warps and crosswise wefts) go criss-cross in the simplest of pattern, at right angles to each other. But in others, they are woven in so many interesting patterns – artistically, decoratively. Do you know that the fabric factories can come up with a mind-boggling number of weave structures?

But the basic weave types that mix and match to form all the others are – plain, twill and satin weaves. They are given interesting variations or they are combined with each other to form different woven patterns

## Types of weave design

There are three basic weave types; plain, twill, and satin. Another way to create a more subtle design is by changing the weave. For a plain weave every other warp thread is raised before the weft is passed through the shed. In order to produce different weaves, a different combination of warp theads is raised. For example, you can raise two adjacent warp threads, in other words two up and two down across the warp, called an irregular basket weave. This produces a fabric which feels slightly thicker and is less prone to creasing.

Another classic weave, which we also have on offer, is called twill. There are a number of different styles of twill. Generally a twill weave is notable for its diagonal lines. In order to weave twill the loom requires at least 3 pedals with each pedal raising different warp threads. So instead of alternating between two pedals, the weaver will use a repeating pattern of the pedals. For example; pedal 1 then 2, then 3. Or alternatively 1,2,3 followed by the reverse order: 3, 2, 1. There are many variations, with varying numbers of pedals. The resulting designs vary from zig zags (called herringbone) to diamond shapes.

### Plain weave

Plain, or tabby, weave, the simplest and most common of all weaves, requires only two harnessses and has two warp and weft yarns in each weave unit. To produce it, the warp yarns are held parallel under tension while a crosswise weft yarn is shot over and under alternate warps across the width of the web. The weave unit is completed at the end of the second row, when the weft has been inserted over and under the opposite set of warps, thus locking the previous weft in place. Fabric length is increased with the insertion of each succeeding weft yarn. When warp and weft yarns are approximately equal in size and quantity, the finished fabric is balanced and potentially stronger than cloth made of the same kind and number of warp and weft yarns in any other basic weave. Tabby woven with different-sized warp and weft yarns results in such fabrics as taffeta and poplin, in which many fine warps are interlaced with proportionately fewer thick weft yarns to form cloths with crosswise ridges or ribs.

The term extended tabby describes any weave in which two or more warps or wefts, or both, are interlaced as a unit. The group includes fabrics with basketry effects and fabrics with ribs formed by groups of warps or wefts in each shed.

Tapestry weave is a tabby in which a variety of coloured weft yarns is interlaced with the warp to form patterns. It is usually an unbalanced weave, with wefts completely covering a proportionately low number of warps. These cloths are sturdy and compact. Although they are flat and generally do not drape well, they have been used for centuries to make ceremonial and decorative dress and costumes.

Denting plan                             tie plan

Drafting plan

Figure Plain weave

### Twill Weave

Twill weave is distinguished by diagonal lines. The simplest twill is that created by the weft crossing over two warp yarns, then under one, the sequence being repeated in each succeeding shot (pick), but stepped over, one warp either to the left or right. Twills with more warps than wefts floating on the fabric’s face are called warp faced; those with wefts predominating, weft faced. The angle of the twill can also vary.

Twills can be varied by changing the relative number of warps and wefts in each repeat (2:1, 2:3, 3:1, 6:2, etc.); by stepping the repeat in one direction; by breaking the direction of the diagonals formed by the twill at regular intervals; by reversing the direction of the diagonal at regular intervals to form chevrons or lozenges; or by combining several twills or modifying them to create a pattern.

Twills drape better than plain weaves with the same yarn count because twills have fewer interlacing. Twill weaves have been used throughout history in many weights and textures, from wool serges mentioned in medieval French manuscripts to English diapered (diamond patterned) table linens, patterned bed coverlets, and Indian shawls.

Denting plan

Drafting plan                                                                              tie up plan

Twill weave 2/1

Figure twill weave

### 3.2.3 Satin weave

Although satin-weave drafts superficially resemble those of twills, satin weave does not have the regular step in each successive weft that is characteristic of twills. Thus, there is no strong diagonal line, and the fabric is smooth faced, with an unbroken surface made up of long floating warp yarns. A true satin must have at least five warp and weft yarns in each complete weave repeat and thus requires at least five harnesses. Most satin fabrics are made of smooth, lightly twisted yarns that heighten the effect of light unbroken by visible crosswise bindings. The limited number of interlacing allows the weaver to use a proportionately large number of warp yarns and thus produce a heavy textured cloth that can be arranged in smooth, shadowed folds. Satins, having long floats, are susceptible to the wear caused by rubbing and snagging and are, therefore, generally regarded as luxury fabrics.

### Jacquard weave

This is a beautiful weave with intricate and colourful designs woven into the fabric. This weave is produced on a jacquard loom.

The weave results in a fabric which is strong and lustrous and has a luxurious look and feel. Jacquards can be made one color or a mix of colors and can be used to create simple as well as complex patterns.

# Unit Four: Weaving process

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Weaving Loom Operation - Type of weaving loom - Mechanism of weaving motion - Materials and tools for Handloom weaving operation - Warp tension adjustment - Weaving faults This unit will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Operate weaving loom - Carried out and identify weaving mechanisms - Use proper tools and materials - Maintained warp tension - Check weaving faults, of a fabric

## Weaving Loom Operation

Weaving is a process of fabric production in which two distinct sets of yarns are interlaced at right angles to each other to form a fabric or cloth. The lengthwise yarns are called the warp yarn and the widthwise yarns are called the weft yarn.

There are three main actions: shedding, picking and battening; and these are common to all types of weaves. Let’s start with the plain weave: that’s the weave that you’ll find in most of our selection of fabrics. The loom has two pedals (also called treadles), in the pit at the feet of the weaver. The pedals are attached to harnesses which in turn have heddles (or heals) attached to them. The harnesses hang above the warp and the heddles are suspended from them. The heddles are loops of string which have a single thread from the warp running through them. When the weaver pushes down on a pedal it raises the harness with the attached heddles which in turn raise the warp threads. Imagine that the threads from the warp are numbered. One of the two pedals raises the even numbered warp threads and the other pedal raises all the odd numbered warp threads. Pressing down on a pedal is called shedding because it creates a shed, a vertical space, between the raised and unraised warp threads. Once the shed has been opened, the weaver then has to pass the weft through it.

The weft thread is spun on a quill, a type of spool, which is then placed inside a shuttle. The shuttle is a wooden container which is pointed at both ends, like the hull of a canoe, to allow it to slide smoothly across the warp. In the past the weaver would have pushed the shuttle through the shed by hand. These days they use what is called a fly shuttle. There are two small loose boxes at each end of the weft which the shuttle fits into. These boxes have strings attached to them which are joined together in the middle of the loom. By holding the two strings and jerking to either the left or the right, the weaver can propel the shuttle across the width of the warp into the box on the opposite side. This action is called picking.

The next job is to maintain the tightness of the weft. This is done through battening. The tool to

perform this action is called a reed. It looks a bit like a comb. The reed is positioned across the weft, perpendicular to the warp. And it holds each thread of the warp apart. After each picking the weaver pulls the reed towards themselves. Battening ensures that the weft, after it is propelled across the warp, is tightly pressed up against the previous weft thread and ensures that ultimately the fabric ends up strong and tightly woven

Selvedge:  The length wise running edges of woven fabric are known as selvedges. It prevents unraveling of warp yarns.

## Types of weaving loom

Weaving can be classified in to different categories:

- Based on driving power of the loom basically classified in to two;

- Handloom and

- Power loom.

- Based on weft carriage devices;

- Shuttle loom and

- Shuttles loom

- Further, Shuttles loom classified based on weft insertion mechanism:

- Projectile

- Rapier

- Air jet

- Water jet

- Based on shedding mechanism;

- Tappet

- Dobby

- Jacquard

Figure 4.1: types of loom

### Handloom

A hand loom is a simple machine used for weaving.  In a wooden vertical -shaft looms, the heddles are fixed in place in the shaft. This loom is powered by hand. The warp threads pass alternately through a heddle, and through a pace between the heddles  (the  shed),  so  that  raising  the  shaft  raises  half  the  threads  (those  passing through  the  heddles),  and  lowering  the  shaft  lowers  the  same  threads the  threads passing through the spaces between the heddles remain in place. It is a manual operating system.

Shedding is done by pressing the treadles pedal and picking and beating is done manually.

Hand looms yield less production compared to power loom.

### Operating Handloom

Attending to Warp Break:

- Find out broken warp ends.

- Find out the location of the broken end by visual examination.

- Mend the broken warp end in the sized beams with the thrums of the same count of

- the sized beams, using " weavers ' knots"

- Draw the mended warp yarn through the heddles properly as per the drawing order

- prescribed.

- Draw the mended warp yarn through the beater properly as per the denting order

- prescribed.

- See that the sley has been brought to the back centre.

- See that the shuttle is inserted fully in the shuttle box.

- Run the loom by using proper hand and foot movements on the appropriate parts of the loom designed for the purpose.

Attending to Weft Break:

- See that the beater is brought to the back centre.

- Take out shuttle from shuttle box.

- Find out the broken pick. Check whether the pick has covered halfway or less.

- Take out the broken pick.

- Carefully do the piecing of the weft yarn with the shuttle yarn (Do not do knotting).

- See that the shuttle is inserted fully in the correct shuttle box.

- Run the loom by using proper hand and foot movements on the appropriate parts  of the loom designed for the purpose.

## Mechanism of Weaving motion

4.3.1 Primary weaving operations:

In order to interlace warp thread and weft yarn to produce a fabric on any type of loom, three operations are necessary, primary motion.

Shedding:

Motion which moves the heald frames up and down in order to separate the warp sheet in to two layers  to make an opening for the weft yarn to pass through.

Shedding is done by three devices:

- Tappet

- Dobby

- Jacquard

Picking:

The passage of the shuttle or other device across the loom to put a weft yarn in the shed. It is a motion through which weft yarn (pick) is put across the width of the fabric.

Picking motions differ from each other depending upon type of loom.

In shuttle looms mainly two types of picking systems are used:

- Over-picking System is used in conventional powerlooms where picking stick operates over the sley of the loom.

- Under-picking system is used in upgraded shuttle looms called Automatic/Semi automatic looms.

In shuttleless looms different types of picking systems are being used such as Gripper, Rapier, Air-jet, Water-jet etc. In fact the nomenclature of shuttleless looms has been developed on the basis of their picking systems.

Beating up:

The pushing of each weft yarn up to the fell of the cloth by the reed, after the shuttle has moved through the shed.

Type of beating systems used also differ as per type of loom.

- In shuttle looms (plain, automatic and semi automatic) crank beating system is used.

- In shuttleless looms mostly for high speed operation cam beating system is used whereas for the machine with slow speed (especially slow speed rapier loom) crank beating system is preferred.

    - Secondary Motion

Two additional operations are important if weaving is to be continued:

Let-Off Motion

It is a motion, which lets the warp-sheet unwound at the rate at which woven cloth is wound on the cloth roll. It also ensures that warp ends are controlled at the optimum tension during weaving. There are mainly two type of let-off motions.

Negative let-off motions

Positive let-off motion

Take Up Motion

It is a motion that wound woven cloth on a wooden or aluminum roller which is known as cloth roller. This motion determines the PPI in the fabric.

    - Auxillary Motion

After the first two operations; primary and secondary motion, weaving process is completed in handloom weaving fabric production. However, some motions are important to improve product quality. These are:

- Temple motion

- Shuttle changing

- Warp tension roller adjustment

## Warp tension

The warp must be under tension to permit weft insertion and fabric construction. The increase in the tension avoids stressing heavily the yarns during the reed beat-up, reduces their sticking together during shedding especially when weaving yarns with poor elasticity and with low hairiness, facilitates the separation of the interlaced or glued yarns and the passage of the knots through the reed. The tension might however increase the tensile stress on the warp threads and consequently lead to a higher number of broken ends. On the other hand the reduction in the tension results into a lower yarn breakage rate and also  onto a lower friction of the threads against the heald frames. In certain cases it could cause however difficulties in obtaining the desired weft.

## Fabric defects:

Defect is an unwanted structure on the fabric due to many reasons. The following are

the some of the type of the fabric defects e.g. missing ends, missing picks, reed mark,

double end, weft crack, weft bar, temple mark, starting mark, float, slubs etc.

4.5.1 The most common fabric defects due to warp are:

Faulty thread: a thread or pieces of thread which are coarse, fine, irregular owing to higher or lower twist or to other twist direction, of different colour, with two or three ends;

Missing thread: a thread or pieces of ground or effect threads which are missing in the fabric weave;

Tight/slack thread: a thread or pieces of thread which are tighter or slacker than the other pieces/threads;

Incorrectly woven yarn: a thread which in some parts only of the fabric is not interlaced in the standard way

Broken warp: small pieces of cut or missing warp thread

Reversed thread: crossed, exchanged threads or thread pieces

Warp stripes: one or more faulty threads giving rise to zones of different aspect; it can be due to scraping or rubbing from members of production machines or to inaccurate reeding;

    - The most common fabric defects due to weft are:

Faulty weft: a weft or pieces of weft which are coarse, fine, irregular (slubs, etc.), twisted reversed, with different twist, of different colour, double weft;

Missing weft: weft or pieces of weft missing in the fabric weave;

Tight/slack weft: a weft or pieces of weft which are tighter or slacker than the other pieces/wefts;

Incorrectly woven weft: a weft which in some parts only of the fabric is not interlaced in the standard way;

Cut wefts: short pieces of cut wefts;

Weft bars (starting marks): visual light/dark effect in weft direction due to higher or lower weft density caused by the weaving machine.

# Unit Five: Product Quality

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Quality standards - Identification of damaged work - Work place safety procedures This unit will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Confirm the specified quality standards - Identify and rectify damaged work - Implement work place safety procedures.

## Woven Fabric Quality Standards

Quality means, the standard of something as measured against other things of a similar kind; the degree of excellence of something. Quality issues are the main factor for an industry to sustain. It can hinder the growth of the industry. Because, no buyer will buy quality less product. Here, one thing should be cleared that, quality in textile or garments sector means buyer requirement.

Fabric is a get together of yarn which is made by interlacing or interloping. knitting and woven are the two fundamental class of fabric. We will discuss the woven fabric quality parameter which alludes diverse test method of fabric, for example, EPI and PPI tally, GSM, quality and so forth.

The quality issues of woven fabric are:

- EPI & PPI

- GSM

- Shrinkage

- Skewness

- Bowing

- Tensile strength test

- Tear strength test

- Pilling test

- Elongation & Recovery

## Identification or quality control of fabric defects

The quality control on the fabrics is carried out on a special inspecting machine, equipped with

special lamps which facilitate the defect detection by the operator, marks them with labels of

different colours according to the fault type and importance.

Depending on the number of faults and on their importance, the fabric pieces can be classified as

standard (in respect to quality specifications) or can be subjected to a more or less serious

degrading with consequent compensations to the customers or with the sale of the fabric at a

reduced price.

Various defects can arise during the stages of weaving preparation (warping, sizing, threading-in

into the heddles and into the reed) as well as during weaving itself. It is therefore important to

regulate accurately the various devices of the weaving machine and to understand how to act in

case of anomalous operating situations which create defects and/or reduce weaving efficiency.

There are some quality parameters of woven fabric:

- Dimensional characterstics

- Length

- Width

- Thickness

- Weight of fabrics

- Weight per unit area

- Weight per unit length

- Threads per inch of fabric

- Ends per inch

- Picks per inch

- Yarn count

- Warp count

- Weft count

- Crimp

- Warp crimp

- Weft crimp

- Handle

- Stifness

- Drape

## Workplace safety procedures

Textiles are an integral part of our lives, from the clothes we wear to the fabrics that cover our sofas, floors, windows, beds, and more.

And the textile industry in America, is subject to important workplace health and safety regulations specific to textile manufacturing.

Workplace Observations and Incident Reporting

Textile companies need to ensure that employee exposure to certain chemicals doesn’t exceed safe levels. For example, several of the chemical dyes commonly used in coloring fabrics can cause cancer. There are ways to mitigate the danger though, and that’s where workplace observations and incident reporting come in.

Solutions may be as easy as donning personal protective equipment (PPE) and clothing and eyewear or as complex as measuring exposure to certain chemicals by taking air samples at regular intervals. Encouraging employees to report incidents through an easily accessible EHS software application can deliver appropriate responses more quickly. And through investigation and analysis of incidents, revisions can be made to company protocols that reduce the chances of similar incidents happening again, thereby improving everyone’s safety.

### Monitor and Track Industrial Hygiene

Health and safety procedures can go beyond best practices like ventilation and chemical exposure standards. They may also dictate ongoing monitoring and assessment rules, such as is required when cotton dust is present in a workplace. Industrial hygiene software can help set up and maintain a schedule of exposure assessment and monitoring, as well as recording and reporting results as needed. Risk assessments built into EHS systems can flag processes or procedures that need revision to ensure safety.

### Manage Training Effectively

Tips for minimizing and mitigating exposure, harm, and liability in the textile industry include familiarizing your team with local, state, and federal regulations, as well as industry standards. In addition, coming up with an EHS training protocol ensures that rules and regulations are learned, understood, and adhered to.

Make sure that procedures are in place for workplace accidents and safety violations, both those that cause active and immediate harm and those that are merely infractions of the rules. When training is developed and delivered, it should be documented in your EHS application. When ongoing training is carried out, document these, too.

Include drills and practices in your EHS processes and procedures. Establish a chain of command and ensure that everyone along the chain of command understands their role and responsibilities in addressing workplace health and safety incidents

### Fulfill Regulatory Requirements

Include in your training any reporting requirements to regulators; know to whom you should report what, when, and how. For better workflow processes, use EHS software to document this information. You can also use it to document employee training, and push new programs to those working the front lines. This will help fulfill regulatory and internal requirements and streamline your own reporting and audits.

# Unit six: Handloom weaving related operations

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - raw materials - Weaving faults and Cause and remedies - Check Quality This unit will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Identify raw materials - Identify Weaving faults and Cause and remedies of weaving faults - Check Quality

## Raw materials

Cotton, silk, wool, and linen are the most popular choices of raw material for handloom weaving. Each weavers uses different raw materials for its unique handloom products.

Yarn is a long continuous length of interlocked fibers. The raw material is gently rolled with palm to form a loosely interlocked cylindrical bunch known as a sliver. This loosely interlocked sliver is then spun on a yarn to make it compact and fine. The spun cotton yarns are braided into skeins and sent for dyeing.

## Weaving faults and Cause and remedies

Faults can  be  reduced  by  using  good  quality  of  yarn,  good  weaving  preparations; proper loom settings at various stages of fabric manufacturing

Back of Loom (Warp Alley)

Slubs: A thick place in the warp: slubs in the warp yarn sheet can cause problems when passing through the heald wires or beater.  Once spotted it is the weaver’s responsibility to remove it, to avoid warp breaks if the slub does not pass the reed; or to avoid a fault if the slub goes into the cloth.

Extra end: Guide it through the guide eyes to the winding device.

Missing end: Take the nearest positioned extra end and guide it through the guide eye s to the missing end position.

Crossed end: To be corrected by the weaver.

Thick end or wrong yarn count: Take out,  guide to the winding device, identify the end as incorrect with a  label, and replace with a normal end from the extra end reserve.

Stuck ends / sizing fault: Separate the ends with the help of the guides.

Spare end bobbin: The extra ends provided on the warp roller need to be guided through the guides provided on the spare end bar, to the side of the loom and then wind onto the spare end bobbin.  These need to be kept tidy otherwise a tangled mess will quickly result. When the bobbin is full it needs to be stripped.

Fluff and fly: When  pieces  of  fluff  or  fly  have  settled  on  the  warp  they  should  be  removed immediately to prevent them from being woven in. Fluff and fly  accumulating on loom  should be removed before it becomes detached  and gets woven  into  the cloth.

Waste / wild yarn: Extra  piece  of  yarn,  which  have  either  been  left  on  warp  roller/  sheet  or  have dropped onto a loom, remove them immediately before they become entangled or woven in.

Fabric Defect Tables

| Name | Appearance | Cause | Prevention |
| --- | --- | --- | --- |
| Thick Place in weft Direction | Bars of denser woven fabric across cloth | Cloth roll take-up and let off of warp rolls not done properly | Cloth inspection. |
| Floating End | Un-woven warp end | Broken heald. cloth inspection | Cloth inspection |
| Slubs. | Thick lumps of | Faulty weft yarn. Not removing broken weft correctly | Cloth inspection |
| Wrong Dent. | Thin line warp | End or ends drawn into the | Know correct denting order. |
|  | Irregular | End or ends drawn into the |  |

## 6.3 Check Quality

Front of Loom (Cloth Alley)

1.  Cloth Quality

Short picks: Is the weft being inserted properly?

Weft bars: Is this a variation of weft; or improperly operated take-up of woven fabric or left off dead weight?

Uneven yarn: Has the weft quality deteriorated and the pirn needs to be changed?

Broken pick: Has the weft been inserted for the whole width of cloth, either breaking in its

insertion or not being held at the receiving side?

Double end: Two ends weaving as one in the same heald break out the extra end.

Wrong draft: An ends have been inserted into the wrong heald eye, resulting in a break

in the cloth pattern.

Wrong dent: An end has been drawn incorrectly in the reed resulting in a warp line down the

cloth or a break in the cloth pattern.

Selvedge: Is the selvedge complete and correct, resulting in a correct edge to the fabric? Is

the selvedge construction correct?

Reed Marks: Is there any warp way lines caused by a damaged beater?

Self-check-1

# Operation sheet-1

Operation Title: xxxxxxxxxxxxx

Instruction: xxxxxxxxxxx

Purpose: xxxxxxxxxxxxxxxxxx

Required tools and equipment: xxxxxxxxxxxxxxxxx

Precautions: xxxxxxxxxxxxx

Procedures:

Step 1. xxxxxxxxxxxxxxxxxxxxxxxx

Step 2. xxxxxxxxxxxxxxxxxxxxx

Step 3. xxxxxxxxxxxxxxxxxxx

Quality criteria:

| # LAP Test | Practical Demonstration |
| --- | --- |

Name: __________________________                                Date: ____________

Time started: ____________________ 		          Time finished: _______________

Instruction I: Given necessary templates, tools and materials you are required to perform the following tasks within 10 hours.

Task 1: xxxxxxxxxx

Task 2: xxxxxxxxx

Task 3: xxxxxxxxx

# Reference

- Bentley, M., Peerenboom, C. A., Hodge, F. W., Passano, E. B., Warren, H. C., & Washburn, M. F. (1929). Instructions in regard to preparation of manuscript. Psychological Bulletin, 26(2), 57–63. https://doi.org/10.1037

WEB Address

- http://www.qualitygurus.com/courses/mod/forum/discuss.php?d=1557

# Developers Profile

| No | Name | Qualification (Level) | Field of Study | Organization/ Institution | Mobile number | E-mail |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |